Texas  FFA  State  Nursery / Landscape  Career Development Event

General  Knowledge  Examination  Question  Bank

Fifty (50) questions for the General Knowledge Examination of the Texas FFA State Nursery/Landscape Career Development Event will be selected from the following list.

1.	PVC pipe for irrigation is joined by connectors/couplings that are                     to the pipe.

- Clamped

- Glued

- Soldered

- Welded

2.	Similar to the textural appearance of plants,                      can be used to create spatial illusions in a perennial garden.

- Plant growth rate

- Color

- Shape

- Staking

3.	To preserve the germination ability of most seeds, store them in a:

- Warm, dry environment

- Cool, dry environment

- Cool environment after scarification

- Warm, moist environment after stratification

4.	___	__ drawings show an object as seen by the eyes at eye-level.

A.	Plan

B.	Section

C.	Perspective

D.	Base map

5.	Fireblight is a disease that most often affects susceptible plants:

- During the winter

- In recently installed landscapes

- During periods of rapid succulent growth

- In landscapes over four years old

6.	The soil pH level primarily controls:

- Soil temperature

- Activity of soil-borne diseases

- Moisture absorption by roots

- Availability of essential plant nutrients

7.	Three types of ground patterns used in landscape design are 		.

A.	Curved-line, arc-and-tangent, straight-line

B.	Curved-line, straight-line, squiggle-line

C.	Curved-line, arc-and-tangent, squiggle-line

D.	Straight-line, squiggle-line, arc-and-tangent

8.	Training plants into three-dimensional ornamental forms, such as animals, is called:

- Topiary

- Espalier

- Heading back

- Bonsai

9.	Clay soils require:

- A longer time for absorption of irrigation water than sandy soils

- A shorter time for absorption of irrigation water than sandy soils

- About the same time for absorption of irrigation water as sandy soils

- More or less time for absorption than sandy soils, depending on the pH of the clay

10.	The approximate range of average annual minimum temperature for Plant Hardiness Zone 5 is:

- –20 to –10 degrees

- 10 to 20 degrees

- –30 to –20 degrees

- 20 to 30 degrees

11.	Fertilizer recommendations are often given in pounds of nitrogen per thousand square feet.  How many 50 pound bags of 10-10-10 fertilizers should be applied to an area 50 X 100 ft. if the recommendation is for 2 pounds of nitrogen per 1000 sq. ft.?

- 1

- 2

- 4

- 7

12.	Plants that have been clipped or pruned into two-dimensional forms are:

A.	Espalier

B.	Topiary

C.	Balanced

D.	Dynamic

13.	A transplanted tree or shrub should be set into the ground:

- 50 percent deeper than it was originally planted

- So that the top roots are exposed to the air

- With the top of the container slightly covered

- At the same depth at which it was originally growing

14.	Which of the following is true about lady bug beetles?  They:

- Often feed on vegetable plants

- Sometimes sting gardeners

- Eat aphids

- Are bright green and blend-in with the foliage

15.	Plants that use less water and are tolerant of dry landscape sites are called:

- Epiphytes

- Xerophytes

- Monoecious

- Dryophytes

16.	Damping-off is caused by:

- Fungi

- Bacteria

- Nematodes

- Virus

17.	Which of the following is a disadvantage of using a time clock to control an irrigation system?

- When there is a power outage, the irrigation system runs continuously.

- The plants are watered at set times, whether they need watering or not.

- Time clocks are very expensive.

- Time clocks are difficult to set.

18.	Which of the following is a group of terms related to landscape design principles?

- Stratify, random, curvature

- Focalization, proportion, simplicity

- Hogarth, symmetrical, curvature

- Thresh, rounding, complexity

19.	The Oak Leaf Nursery advertised a group of trees with a trunk caliper of 3 inches.  In the trade, it is assumed that this measurement was taken:

- 18 inches above ground level

- At the base of the rootball

- 6 inches above ground level

- At breast height (4½ feet above the rootball)

20.	One function of stomates is to:

- Give off water vapor

- Prevent disease

- Photosynthesize

- Produce pollen

21.	A friable soil is one that:

- Is rich in N, P, and K

- Is loose and of good structure and texture

- Is high in clay particles

- Requires humus and fertilizer to be productive

22.	Plants usually do not grow well in poorly drained soils because:

- Oxygen content is low in poorly drained soils

- The organic matter content of poorly drained soils is too high

- Poorly drained soils warm too quickly in spring

- Plant roots become engorged with water in poorly drained soils

23.	To make the living area of a landscape appear larger, a designer should include plants that feature:

- Warm-colored flowers such as reds, oranges, yellows

- Cool-colored flowers such as blues, purples, lavenders

- A combination of warm- and cool-colored flowers

- All white flowers

24.	Which of the following turf areas is mowed to the lowest height?

- Commercial lawn

- Golf fairway

- Home lawn

- Golf green

25.	In plants,                      is the process of converting stored energy into energy for plant growth.

- Transpiration

- Photosynthesis

- Respiration

- Pollination

26.	A landscape designer uses many 		 in developing the preliminary design for a project.

A.	Perfect plans

B.	Bubble diagrams

C.	Site surveys

D.	Problem statements

27.	Fairy ring is caused by:

- Fungi

- Grubs

- Pollen

- Nematodes

28.	An “x” in the botanical name of a plant means the plant:

- Is a true hybrid

- Comes true to type from seed and vegetative propagation

- Comes true to type from only seed propagation

- Has been grafted

29.	An example of an organic material is 		.

A.	Pea gravel

B.	Perlite

C.	Leaf mold

D.	Vermiculite

30.	Which of the following is microscopic?

- Thrips

- Spider mites

- Nematodes

- Aphids

31.	The loss of water in the form of vapor from the surface of plant leaves is referred to as:

- Transpiration

- Respiration

- Oxidation

- Condensation

32.	The basic underlying principle of using a mist propagation system is that it:

- Keeps the propagation medium uniformly moist and supplies moisture to the base of the cuttings

- Keeps the plant material cool

- Cools the propagation medium to promote root formation and development

- Increases humidity around the cutting and reduces transpiration

33.	__________ is an example of a parasitic plant.

A.	Mistletoe

B.	Rust

C.	Bacteria

D.	Mildew

34.	Low-growing landscape plants should:

A.	Be placed in front of intermediate growing plants

B.	Be placed behind intermediate growing plants

C.	Not be used in the landscape

D.	Be planted under large trees

35.	Which signal word for pesticide labels indicates the lowest level of toxicity?

- Danger

- Warning

- Careful

- Caution

36.	In a landscape design, properly located plants and structures.

A.	Can reduce heat intensity

B.	Need to be constantly pruned and maintained

C.	Block the view of the front door

D.	Are not essential for a good landscape design

37.	Which of the following would be most suited for a hot, dry, steep bank in the landscape?

- Creeping juniper

- English ivy

- Dogwood

- Fescue

38.	A shade loving border plant is:

- Marigold

- Impatiens

- Zinnia

- Chrysanthemum

39.	Soft pencil leads are indicated by the letter 		.

A.	H

B.	B

C.	H1

D.	F

40.	A plant pathologist studies plant:

- Insects

- Diseases

- Morphology

- Physiology

41.	A plant that dies to the ground at the end of each growing season but comes back from the same root-stock year after year is most accurately described as a(n):

- Perennial

- Annual

- Biennial

- Herbaceous perennial

42.	Two basic types of pruning are heading-back and thinning-out.  Which of the following is true about these pruning methods?

- Thinning cuts leave behind a good portion of the stem and many axillary buds.

- Thinning cuts are made close to a main branch or ground and few axillary buds are left.

- Heading cuts usually remove entire branches and leave no buds.

- Heading cuts remove a portion of the stem and only terminal buds are left.

43.	Lowering the soil grade around a tree removes many of the tree’s 		 roots.

A.	Tap

B.	Aerial

C.	Feeder

D.	Aquatic

44.	Water availability, temperature extremes, and 		 should be considered when selecting trees for the landscape.

A.	Slope

B.	Topography

C.	Soil type

D.	Buildings present

45.	The four life cycle stages of beetles such as the Japanese beetle are:

- Egg, grub, pupae, adult

- Moth, egg, grub, adult

- Grub, pupae, juvenile, adult

- Egg, nymph, pupae, adult

46.	An important step in properly composting landscape refuse is to:

- Exclude as much air as possible from the compost pile

- Treat each layer with pesticide to control insects and weeds

- Make only small piles to keep the material cool

- Thoroughly moisten each layer of material

47.	A pencil lead that is soft, prints well, erases easily, and tends to smear easily is 		.

A.	9H

B.	3B

C.	2H

D.	F

48.	A large perennial that provides a general effect of mass and grows in excess of five feet is:

- Yarrow

- Primrose

- Pampas grass

- Dianthus

49.	Lawn sprinkler systems are installed so they provide even coverage.  This is accomplished by:

- Spacing the heads so that each sprinkler sprays to the next sprinkler

- Using sprinkler heads with a square pattern throughout the landscape

- Spacing sprinklers based on needs specified in the soil test

- Using one size pipe throughout the landscape

50.	The statement on a bag of fertilizer that lists the percentages of nitrogen, phosphorous, and potassium contained in the fertilizer is called a(n) 		 analysis.

A.	Fertilizer

B.	Micro-element

C.	Chelated-Release

D.	Essential element

51.	Callus is a term usually referring to:

- The tissue on the collar of a limb

- The first tissue to develop on a cutting just before the roots form

- Deformed growth on plant limbs

- A disease of plant leaves and limbs

52.	A                      is a woody perennial that has more than one main trunk.

- Tree

- Flower

- Shrub

- Weed

53.	Mrs. Thompson loves spring flowering bulbs.  Which of the following would you suggest for Mrs. Thompson to plant?

A.	Hyacinthus orientalis cv.

B.	Coleus x hybridus

C.	Lonicera japonica

D.	Gardenia jasminoides

54.	If a two-cycle engine on a weed trimmer uses a 24:1 fuel ratio, you should mix:

- 24 parts gasoline to 1 part oil

- 24 parts oil to 1 part gasoline

- Equal amounts of gasoline and oil

- 24 parts gasoline to 1 part engine cleaner

55.	Soil texture refers to:

- The arrangement of soil particles

- The size of the soil particles

- The ratio of sand to clay in the soil

- The amount of humus in the soil

56.	In nursery production, a “liner” refers to 			.

A.	A plastic bag

B.	A rooted cutting or small seedling

C.	A type of garden tool

D.	A plastic mulch covering

57.	Which of the following is a deciduous vine?

- Chinese wisteria

- Mandevilla

- English ivy

- Rosemary

58.	When planting ground covers grown in compressed peat containers:

A.	Be sure to remove the plant completely from the peat container

B.	Make several cuts down the sides of the container before planting

C.	Leave the container as is

D.	Leave ½ inch of compressed peat container exposed above the soil line

59.	What part of a lawn mower prevents dust from entering the carburetor?

- Air cleaner

- Exhaust chute

- Crankcase

- Hydraulic system

60.	The optimum germination temperature for most bedding plant seeds is:

- 50 to 55ºF

- 60 to 65ºF

- 65 to 75ºF

- 75 to 85ºF

61.	Paul had 400 containers of mature Salvia to wholesale to a landscaper.  His cost of producing the plants, including labor, was $832.  If he adds 30% to the cost of production as his profit, how much will he charge the landscaper per container?

- $0.27

- $2.14

- $2.70

- $4.81

62.	Which of the following statements is true about applying dry fertilizer to a lawn?

- Apply dry fertilizer just after a rain

- Apply dry fertilizer in the early morning when the grass is wet from the dew

- It is a good practice to water-in dry fertilizer applied to a lawn

- Dry fertilizer should only be applied late in the day

63.	The classifications of soil particles based on size are sand, silt, and:

- Clay

- Loam

- Shale

- Gravel

64.	Which fuel is used in chainsaws, string trimmers, and other 2-cycle engines?

- Gasoline

- Fuel oil

- Diesel fuel

- Gasoline and oil mix

65.	Which of the following is the proper recommendation for pruning trees at planting:

- Pruning should be limited to shaping the plant by removing dead or damaged limbs

- Trees should be cut back to at least one-half their size to prevent stress on root systems

- Trees should not be pruned until the third year after planting

- Trees should not be pruned at all – the latest trend is to let them develop naturally

66.	The practice of manually watering a container plant until water comes out of the drainage holes of the container is effective for helping to reduce the buildup of                      in the growing medium.

- Pathogens

- Soluble salts

- Pesticides

- Herbicides

67.	A scale of 1” = 2’0’ means that:

A.	2’ of landscape equals 1 1/2” of blueprint

B.	1” of blueprint equals 30” in the landscape

C.	1” of blueprint equals 2’ in the landscape

D.	5’ of landscape equals 6” of blueprint

68.	The best time to spray a glyphosate is:

- Just before a rain

- When weeds are in active growth

- When weed growth is mature

- Just before weed seeds germinate

69.	Which of the following groups contains only evergreen plants?

- Bald cypress, Purpleleaf plum, Pecan

- Azalea, Ginkgo, Chinese Holly

- American arborvitae, Camellia, Japanese Yew

- Sweet gum, Crape myrtle, Sycamore

70.	Which of the following is a group of small-growing trees?

- Sweet gum, Sycamore, Southern magnolia

- Red oak, Bald cypress, Live oak

- Crape myrtle, Eastern redbud, Japanese black pine

- Pecan, Sweet gum, Ginkgo

71.	When referring to the USDA Plant Hardiness Zone map, one can determine the:

A.	Minimum expected temperature for an area

B.	Maximum expected temperature for an area

C.	Soil characteristics for an area

D.	Humidity and microclimatic changes for an area

72.	One of the best ways to speed up the rooting process of cuttings is to:

- Provide bottom heat

- Keep the soil cool and the foliage warm

- Harden them off by laying them out to dry a few days before sticking

- Always take cuttings after mid-day

73.	Fred was about to drive the tractor down the street to another landscape job when Alan said, “Hey, you had better put that orange triangle on the back of that tractor before you get run-over.”  Alan was referring to the:

- Farm vehicle sign

- Slow-moving vehicle sign

- Emergency vehicle sign

- Wide-load sign

74.	Divide and separate fall blooming perennials:

- In early spring

- In late spring

- Mid summer

- Early fall

75.	Leaf miners lay their eggs:

- On top of leaves

- Inside the leaves

- On the main stem

- On flower buds

76.	An area of the landscape that includes space for such items as a clothes line and garbage cans is the                      area.

- Service

- Public

- Private

- Recreational

77.	The landscape design principle of interconnection:

A.	Is a visual break in sequence

B.	Is the human perception of space and form related to human dimension

C.	Is the overall organization and structure of a design

D.	Occurs when visual components are physically linked together

78.	When trying to save trees on a construction site, cut and fill under the drip line of the trees should be limited to a depth of                      inches.

- 6

- 8

- 18

- 0 (Never cut or fill under the drip line of trees.)

79.				 pruning is a pruning technique concerned primarily with the removal of branches that could lead to damage or weaken a plant’s health and appearance.

A.	Natural target

B.	Random

C.	Canopy

D.	Crotch

80.	Underground stems with scale-like fleshy leaves (leek and onion) are called:

- Tubers

- Roots

- Bulbs

- Stolons

81.	In most plants, a hormone called auxin is produced by the terminal bud.  It moves downward into the shoot and inhibits the growth and development of lateral buds.  This is called:

- The apex auxin complex

- Apical dominance

- Lateral dominance

- Terminal bud inhibition

82.	Some seeds are produced in the fall and remain on or in the ground until they germinate in the spring.  Exposing seeds to the winter cold and moist soil is nature’s method of                     .

- Scarifying

- Stratifying

- Acidifying

- Sporulating

83.	Once they are beneath the bark surface,                      are difficult to control.

- Borers

- Thrips

- Aphids

- Mealybugs

84.	The use of plant materials and landscaping techniques to conserve soil moisture and to reduce water usage is:

- Xeriscaping

- Waterscaping

- Natural landscaping

- Bogscaping

85.	Which of the following describes plants that lose their leaves during the winter?

- Evergreen

- Deciduous

- Coniferous

- Herbaceous

86.	Hardiness refers to:

A.	The strength of the wood of a tree

B.	The quality of being strong and being able to survive in a certain region

C.	The hardness of the soil in which a plant is being planted

D.	The ability of a plant to grow indoors

87.	Hardening-off is a term used in the nursery/landscape industry to mean:

- Allowing herbaceous plants to become woody

- Acclimatizing landscape plants to coldframe conditions

- Coating plants with a protective plastic-type spray prior to transplanting

- Acclimatizing tender plants to harsher, outdoor conditions

88.	Which one of the following plant hardiness zones is colder than Zone 6?

- Zone 10

- Zone 8

- Zone 7

- Zone 3

89.	Many insect traps attract insects with:

- A sex attractant

- Dried flowers

- A strong insecticide

- Fresh flowers

90.	Some vines use 		 to support themselves as they climb.

A.	Tendrils

B.	Aquatic roots

C.	Stems

D.	Tap roots

91.	A                      is a landscape designer’s tool for drawing circles on a landscape plan.

- Scale

- Circle template

- T-square

- Protractor

92.	The following plant grows from a tuber:

A.	Celosia

B.	Impatiens

C.	Japanese honeysuckle

D.	Elephant Ear

93.	The best definition of a surfactant is a:

- Material added to a liquid spray solution to reduce surface tension between the liquid and leaf surface

- Type of soap used to wash off pesticide residue by increasing surface tension between the leaf and residue

- Material to protect leaf surfaces from pesticides by creating a waxy seal over the leaf

- Type of pesticide formulation with an oily consistency for protecting high pressure spray nozzles from wear

94.	Which type of insecticide would be most effective against sucking-type insects?

- Contact

- Systemic

- Dust

- Aerosol

95.	The three major divisions of a soil profile are:

- mulch, clay, and parent material

- topsoil, subsoil, and bedrock

- topsoil, clay layer, and rock layer

- topsoil, subsoil, and parent material

96.	A lawn mower with blades parallel to the surface of the ground is called a(n):

A.	Reel mower

B.	Rotary mower

C.	De-thathing mower

D.	Electric mower

97.	Which of the following groups contains only deciduous plants?

- Cedar Elm, Carya illinoensis, Red Oak

- River Birch, Southern Magnolia, Ilex vomitoria

- Taxus cuspidata, Sycamore, Bradford pear

- Live Oak, Chinese Tallow, Sophora secundiflora

98.	A 50 pound bag of 5-10-15 fertilizer contains                      pounds of nitrogen.

- 15

- 10

- 5

- 2.5

99.	The scientific name for Heavenly Bamboo is:

A.	Ligustrum japonicum

B.	Nandina domestica

C.	Pyracantha coccinea

D.	Raphiolepsis indica

100.	The turfgrass least tolerant of shade is:

- Zoysia

- Fescue

- St. Augustine

- Bermudagrass

101.	Which of the following is not an important environmental requirement for germination to occur?

- Proper temperature

- Oxygen

- Carbon dioxide

- Moisture

102.	Light energy, carbon dioxide, and water enter into the process of photosynthesis, through which:

- Respiration results

- Carbohydrates are formed

- Chlorophyll is formed

- Amino acids are formed

103.	When planting B&B plants:

- The burlap should be completely removed from the ball

- The tie string should be cut from the trunk of the plant

- The burlap should be pulled up and tied securely to the trunk for protection

- The burlap should be allowed to dry completely before planting

104.	The most common synthetic root-promoting chemicals used in rooting hormones are:

- IBA and AAN

- CCA and ACD

- ISO and OSB

- IBA and NAA

105.	There are five common methods of night lighting for landscapes.  They are:

- Bright, dim, subdued, marginal, and transparent lighting

- Walk, silhouette, shadow, up, and down lighting

- Moderate, bright, dim, regular, and flash lighting

- Cross, down, up, reverse, and slope lighting

106.	What information is not contained in the legend on a final landscape plan?

A.	Plant names

B.	Specific notes concerning installation

C.	Plant quantities

D.	Designer’s name

107.	When planting a bare-rooted tree, the tree should be planted:

- at 6 inches above the original soil level

- At 2 inches above the original soil level

- At 6 inches below the original soil level

- Directly even with the original soil level

108.	The road in front of Steve’s house has become traffic congested in recent years.  The view, and especially the noise, bothers the family.  When Steve discussed this problem with a landscaper, the landscaper mentioned berms.  What was the landscaper talking about?

- A special type of wall to block the noise and view of traffic on the road

- His recent fishing trip where he caught several berms

- Trees planted in multiple rows between the house and road

- Tall mounds of soil placed between the house and road to block the noise and view

109.	A propagator in a nursery business is responsible for:

- Starting new plants from seeds, cuttings, grafting, or other means and planning the propagation schedule

- Increasing new accounts by spreading propaganda

- Carrying out an extensive plant breeding program and applying for plant patents

- Making sure the area used for pulling orders of container grown landscape-size plants is well stocked

110.	A glyphosate is a non-selective herbicide that will kill:

- Perennial weeds

- Any green plant that it contacts

- Annual weeds

- Both annual and perennial weeds

111.	A major advantage for producing nursery stock in containers is that it:

- Is less expensive than growing in nursery fields

- Allows for transplanting any time during the growing season

- Requires less water and fertilization than field-grown crops

- Requires less labor than field-grown crops

112.	A method of fertilizer application is 			.

- Sterilization

- Pasteurizing

C.	Side dressing

D.	Leaching

113.	Brown patch, dollar spot, and gray snow mold are all diseases of:

- Azaleas

- Turfgrasses

- Trees

- Evergreens

114.	Which plant is a vine?

- Ficus benjamina

- Pinus thunbergiana

- Ilex cornuta

- Lonicera japonica

115.	If a container-grown nursery crop has been over watered for a long period, a grower might expect to find the roots in the following condition:

- The roots dead or absent at the bottom of the container but healthier in the top portion of the container

- The roots dead or absent at the top of the container but healthier in the bottom of the container

- The roots growing through the drain holes in the bottom of the container

- The roots may be extra thick and bright white in color

116.	Site features to consider when conducting a site survey include:

A.	Topography, client analysis, natural features, and climate

B.	Topography, soil conditions, natural features, and existing vegetation

C.	Natural features, existing vegetation, soil conditions, and client analysis

D.	Natural features, climate, existing vegetation, and client analysis

117.	Food produced in the leaves moves to other parts of a plant through the                     .

- Phloem

- Xylem

- Veins

- Arteries

118.	Which of the following would be the best response if a client calls you in October and wants to know what to do about a 20-foot Japanese pine that has turned completely brown?

- Cut it down.  It is definitely dead.

- Wait until spring and see if it sprouts green needles again.

- Do nothing.  Pine trees shed naturally in the fall.

- Do nothing.  The Japanese pine is just going dormant.

119.	When installing a landscape, which step (from the following list) should be completed last?

A.	Plant the trees and shrubs

B.	Add amendments to the soil

C.	Install the drainage system

D.	Build retaining walls and garden walkways

120.	Cuttings in the rooting bench should be:

- Kept constantly moist

- Kept dry during the day and moist at night

- Fertilized regularly

- Scarified before watering

121.	Examples of cool season grasses include perennial ryegrass, bentgrass, and:

- Bermudagrass

- Fescue

- St. Augustine

- Zoysia

122.	Magnesium, calcium, and sulfur:

- Are essential elements needed for plant processes

- Are micro-elements

- Can be found in incomplete fertilizers

- Are only needed by some plants

123.	A “spreader-sticker” added to a spray solution causes the chemical to:

- Become more diluted in the sprayer

- Better cover the plant surface and resist being washed off by rain and irrigation

- Stick to the diseased areas of the plant and run-off the areas it is not needed

- Penetrate the cuticle to the leaf, allowing more effective sticking power

124.	Informal landscapes:

A.	Contain geometric patterns and perfect balance

B.	Include sheared and sculpted hedges

C.	Are constructed with asymmetrical balance and curved lines

D.	Usually contain straight, not curved, lines

125.	When selecting ginkgo trees for the home landscape, you should choose:

- Male and female trees

- Male trees only

- Female trees only

- Mature female trees only

126.	Functional roles of plants in the landscape do not include:

A.	Screening

B.	Developing the structural framework of the design

C.	Enclosing

D.	Greening up

127.	Professional-grade string trimmers are usually powered by:

- Electricity

- A 4-cycle engine

- A 2-cycle engine

- Diesel fuel

128.	To increase the bushiness of a flowering plant while the plant is young, one should:

- Apply a growth retardant

- Increase the nitrogen in the soil surrounding the plant

- Pinch the stem tips of the plant

- Increase the amount of water the plant receives

129.	Liriope and Ajuga are generally classified as:

- Small flowering shrubs

- Groundcovers

- Small evergreen shrubs

- Semi-evergreen shrubs

130.	The recommended minimum width for a residential walkway in the public area of the landscape is                      feet.

- 3.5

- 6.5

- 5

- 7.5

131.	Which of the following is a major reason for mulching landscape plants?

- Increases anti-aerobic activity in the soil

- Supplies nutrients

- Moderates soil temperature and conserves soil moisture

- Attracts earthworms

132.	A color scheme in a garden that produces a carnival-like effect is referred to as being:

- Monochromatic

- Analogous

- Complementary

- Polychromatic

133.	A group of plants located along the perimeter of a landscape use area is called a 		 planting.

A.	Border

B.	Corner

C.	Foreground

D.	Foundation

134.	Which tree has the finest textured foliage?

- Southern magnolia

- Sweet gum

- Mesquite

- Sycamore

135.	Which of the following serves as a food reservoir for the developing embryo within the seed of a monocot?

- Endosperm

- Radical

- Plumule

- Testa

136.	The term                      is used in horticulture to define plant injury sometimes caused by pesticides.

- Eutrophication

- Expectoration

- Emulsification

- Phytotoxicity

137.	An example of a creeping or prostrate coniferous evergreen is:

A.	Juniperus horizontalis

B.	Lobularia maritima

C.	Abelia grandiflora

D.	Prunus caroliniana

138.	A major advantage for using slow-release fertilizer in container-grown nursery stock is that it:

- Supplies nutrients over a long period of time with one application

- Can easily be leached from the soil if too much is applied

- Can be applied without expensive equipment

- Can be injected through the irrigation system

139.	A plant that remains vegetative during its first season of growth and flowers in the second season is a(n):

A.	Biennial

B.	Annual

C.	Propagule

D.	Perennial

140.	A landscaper in New York calls a wholesaler/grower in Tennessee to inquire about plant material for a landscape job.  One of the trees he asks about is bald cypress.  He wants to know if it is hardy in middle New York.  The grower says, “Let me see, that is Zone 4 …”They are talking about the:

- AAN Hardiness Zone Plan

- UPS Zone Index for safe shipping of plants

- USDA Plant Hardiness Zones

- Critical Zone Hardiness Index

141.	Plowing or tilling a bed or lawn when it is excessively wet may:

- Kill beneficial organisms in the soil

- Damage the structure of the soil

- Change the soil texture

- Stratify the soil

142.	Most landscape plants like the soil pH:

- Slightly acidic

- Strongly acidic

- Slightly alkaline

- Strongly alkaline

143.	In the plant name Ilex x merserveae ‘Blue Princess’:

- The genus is Ilex, the “x” means it is a hybrid, the species is meserveae, and the cultivar is Blue Princess.

- The family is Ilex, the “x” means it is a tetraploid, the genus is meserveae, and the species is Blue Princess.

- The species is Blue Princess, the “x” means that its origin is unknown, the variety is meserveae, and the genus is Ilex.

- The genus is Ilex, the “x” means that it is patented, the species is meserveae, and Blue Princess is the variety.

144.	B&B in the nursery/landscape trade means:

- Balled and burlapped

- Bagged in burlap

- Bare-rooted and bagged

- Burlapped in a bag

145.	When installing a landscape, which step (from the following list) should be completed FIRST:

A.	Plant the trees and shrubs

B.	Add amendments to the soil

C.	Install the drainage system

D.	Build retaining walls and garden walkways

146.	Which insect eats out trails between the leaf surfaces?

- Thrips

- Leaf miners

- Aphids

- Spider mites

147.	Wisteria, clematis, and Japanese honeysuckle are examples of:

- Trees

- Vines

- Shrubs

- Groundcovers

148.	When should soil NOT be tilled?

- When it is wet

- When it is dry

- During late fall

- During spring warm-up

149.	When using heat to sterilize/pasteurize soil, the soil should be heated to:

- 100ºF for 1 hour

- 280ºF for 1 hour

- 200ºF for 1 hour

- 180ºF for 30 minutes

150.	Mr. Roberts told his horticulture class that the temperature for rooting shrubs should be no lower than 70 ºF.  He was referring to the:

- Air temperature above the bench

- Air temperature at bench level

- Temperature of the mist water

- Temperature of the rooting media